- SPDM_CMD

The SPDM_CMD command (user-defined function code 65h) is used to attestation and device authentication via Security Protocol and Data Model (SPDM)  over MODBus..

General SPDM Request/Response Flow

| Response ( PSU ) | Code |
| --- | --- |
| VERSION | 0x04 |
| CAPABILITIES | 0x61 |
| ALGORITHMS | 0x63 |
| CERTIFICATE | 0x02 |
| ERROR (RESPONSE_NOT_READY) | 0x7F |
| CHALLENGE_AUTH | 0x03 |
| MEASUREMENTS | 0x60 |
| CHUNK_SEND_ACK | 0x05 |

Supporting Response command ( PSU )

| Requestor ( PMC ) | Code |
| --- | --- |
| GET_VERSION | 0x84 |
| GET_CAPABILITIES | 0xE1 |
| NEGOTIATE_ALGORITHMS | 0xE3 |
| GET_CERTIFICATE | 0x82 |
| CHALLENGE | 0x83 |
| RESPOND_WHEN_READY | 0xFF |
| GET_MEASUREMENTS | 0xE0 |
| CHUNK_SEND | 0x85 |
| CHUNK_GET | 0x86 |

Supporting Requestor command ( PMC )

- Selected SPDM Messages

- Measurements & Firmware Attestation

All measurements within the PSU shall be made using a secure, cryptographic hashing algorithm that meets the requirements of the Algorithm Support section of this document. The following areas of memory shall be individually measured at boot time with an immutable boot ROM:

| Memory Area | SPDM Measurement Block ID |
| --- | --- |
| Main Application (Secondary side Microcontroller) | 01h |
| Main Application (Primary side Microcontroller) | 02h |
| Third Microcontroller | 03h |
| Fourth Microcontroller | 04h |
| Reserved | 05h-FFh |

The PSU shall respond to a GET_MEASUREMENTS request with signed measurements for each area requested. Section 10.11 of [1] describes the protocol for this Request/Response pair. PSU do not require support for recomputing all measurements without requiring a Reset

- References

[1] Security Protocol and Data Model (SPDM) Specification, Version 1.2.0, 2021-12-14, IETF, Document Identifier: DSP0274, https://www.dmtf.org/content/dmtf-releases-security-specification-12